Method

实验中使用的设备为一个约含0.55torr 氙气的 thyratron，一个3V的电源，用于产生稳定电子束，一个0至50V可调节电源，用于驱动电子束穿透气体并在集电极产生测量电流，和一个预制的电路盒，用于测量实验数据

将3V电源打开，等待五分钟，确保灯丝充分加热以保证数据的准确性。然后从0开始逐渐增大加速电压直13.8V，并记录下V-Vs位于0V-3.5V时的值和对应的Vs和Vp的值，在关键电离跃变处缩小电压步进，并对每个测点进行多次读数取平均以减少随机误差

将thyratron浸泡于装满液氮的罐子中，关闭3V电源，然后将其静置十分钟以充分冷却thyratron，然后再次打卡灯丝，等待五分钟。再次从零增大加速电压，并记录对应的V-Vs,Vs和Vp

The 3 V DC filament supply was turned on and allowed to stabilize for 5 minutes to ensure the filament reached thermal equilibrium and to eliminate thermal drift. The 0–30 V DC supply was then used to increase the accelerating voltage V-Vs from 0 V in 0.1 V increments up to 3.5 V; at each step the shield voltage Vs ​, plate voltage Vp and V−Vs ​ were recorded. Upon approaching the xenon ionisation region (≈11–13 V), the voltage increment was reduced (e.g. to 0.05 V) and three consecutive readings were taken at each point and averaged to reduce random error. Throughout this scan, Vs was monitored and kept below 3 V to protect the thyratron.

Next, the filament supply was switched off and the thyratron’s tip was slowly immersed in liquid nitrogen for 10 minutes to condense and freeze the xenon gas. The filament supply was then re-energized and allowed another 5 minutes to re-stabilize thermally. Under these cryogenic conditions, the accelerating-voltage scan from 0–3.5 V (0.1 V steps) was repeated, recording V-Vs ​, Vs ​and Vp ​at each point while ensuring the tube tip remained submerged. Identical measurement procedures and averaging were used to guarantee direct comparability between ambient-temperature and low-temperature data.

将实验中得到的plate electrode两端的

实验中Rp阻值约为10K欧姆,根据实验得到的电压，分别计算出浸泡液氮和未浸泡液氮时不同电压下通过plate electrode的电流，可以得到如下图像: